STM32(Blue Pill ) code for the Feeding and temperature system

#include<Servo.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

Servo myServo;

// Set the LCD address to 0x3F for a 16 chars and 2 line display

//scl-b6 sda-b7

LiquidCrystal_I2C lcd(0x27, 16, 2);

int servoPin = PA0;

int DSPIN = PA8;

int relayHeater=PB15;

double temp;

int servofeed=PB12;

int openclose;

int flag=1;

void setup()

{

myServo.attach(servoPin);

myServo.write(0);

pinMode(servofeed,INPUT_PULLUP);

pinMode(relayHeater,OUTPUT);

digitalWrite(relayHeater,LOW);

lcd.begin();

lcd.backlight();

lcd.setCursor(0,0);

lcd.print(" Starting....... ");

lcd.setCursor(0,1);

lcd.print(" --WelCome-- ");

delay(2000);

lcd.clear();

}

void loop()

{

temp = TempRead();

temp  = temp * 0.0625; // conversion accuracy is 0.0625 / LSB

if(temp>-10 && temp <=110){

displayTemp();

}

else{

delay(1000);

}

openclose=digitalRead(servofeed);

if(openclose==1 && flag==1 ){

myServo.write(0);

delay(500);

myServo.write(60);

delay(500);

myServo.write(0);

flag=0;

lcd.setCursor(0,1);

lcd.print("Fish Feeded");

delay(1000);

}

if(openclose==0 && flag==0){

flag=1;

}

delay(1000);

}

void displayTemp(){

lcd.clear();

lcd.setCursor(0,0);

lcd.print("Temp:");

lcd.print(temp,2);

lcd.print((char)223);// to print Degree symbol

lcd.print("C");

if(temp<=28){

lcd.setCursor(0,1);

lcd.print("Heater ON");

digitalWrite(relayHeater,HIGH);

}

else{

lcd.setCursor(0,1);

lcd.print("Heater OFF");

digitalWrite(relayHeater,LOW);

}

delay(1000);

}

boolean DS18B20_Init()

{

pinMode(DSPIN, OUTPUT);

digitalWrite(DSPIN, HIGH);

delayMicroseconds(5);

digitalWrite(DSPIN, LOW);

delayMicroseconds(750);//480-960

digitalWrite(DSPIN, HIGH);

pinMode(DSPIN, INPUT);

int t = 0;

while (digitalRead(DSPIN))

{

t++;

if (t > 60) return false;

delayMicroseconds(1);

}

t = 480 - t;

pinMode(DSPIN, OUTPUT);

delayMicroseconds(t);

digitalWrite(DSPIN, HIGH);

return true;

}

void DS18B20_Write(byte data)

{

pinMode(DSPIN, OUTPUT);

for (int i = 0; i < 8; i++)

{

digitalWrite(DSPIN, LOW);

delayMicroseconds(10);

if (data & 1) digitalWrite(DSPIN, HIGH);

else digitalWrite(DSPIN, LOW);

data >>= 1;

delayMicroseconds(50);

digitalWrite(DSPIN, HIGH);

}

}

byte DS18B20_Read()

{

pinMode(DSPIN, OUTPUT);

digitalWrite(DSPIN, HIGH);

delayMicroseconds(2);

byte data = 0;

for (int i = 0; i < 8; i++)

{

digitalWrite(DSPIN, LOW);

delayMicroseconds(1);

digitalWrite(DSPIN, HIGH);

pinMode(DSPIN, INPUT);

delayMicroseconds(5);

data >>= 1;

if (digitalRead(DSPIN)) data |= 0x80;

delayMicroseconds(55);

pinMode(DSPIN, OUTPUT);

digitalWrite(DSPIN, HIGH);

}

return data;

}

int TempRead()

{

if (!DS18B20_Init()) return 0;

DS18B20_Write (0xCC); // Send skip ROM command

DS18B20_Write (0x44); // Send reading start conversion command

if (!DS18B20_Init()) return 0;

DS18B20_Write (0xCC); // Send skip ROM command

DS18B20_Write (0xBE); // Read the register, a total of nine bytes, the first two bytes are the conversion value

int temp = DS18B20_Read (); // Low byte

temp |= DS18B20_Read () << 8; // High byte

return temp;

}

ESP8266 Code for the Inlet outlet and air pump

//-------------------

#define BLYNK_TEMPLATE_ID "TMPL3ln0RoP1Y"

#define BLYNK_TEMPLATE_NAME "smart fish task"

#define BLYNK_AUTH_TOKEN "7hIV-UrdoDx9nIjN69EzILinZiWueGzi"

// Comment this out to disable prints and save space

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

char auth[] = BLYNK_AUTH_TOKEN

// Your WiFi credentials.

// Set password to "" for open networks.

char ssid[] = "JioFiber-1spqrsh";

char pass[] = "om123456";

// define the GPIO connected with Relays and switches

//#define RelayPin1 5  //D1

//#define RelayPin2 4  //D2

#define RelayPin1 5  //D1

#define RelayPin2 4  //D2

#define RelayPin3 D4 //D5

#define RelayPin4 D8 //D6

#define SwitchPin1 10  //SD3

#define SwitchPin2 0   //D3

#define SwitchPin3 13  //D7

define SwitchPin4 3   //RX

#define wifiLed   16   //D0

#define VPIN_BUTTON_1    V0

#define VPIN_BUTTON_2    V1

#define VPIN_BUTTON_3    V2

#define VPIN_BUTTON_4    V3

#define TRIGPIN    D6  //D6

#define ECHOPIN    D5  //D5

int toggleState_1 = 0; //Define integer to remember the toggle state for relay 1

int toggleState_2 = 0; //Define integer to remember the toggle state for relay 2

int toggleState_3 = 0; //Define integer to remember the toggle state for relay 3

int toggleState_4 = 0; //Define integer to remember the toggle state for relay 4

int wifiFlag = 0;

float duration;

float distance;

BlynkTimer timer;

void checkBlynkStatus() { // called every 3 seconds by SimpleTimer

bool isconnected = Blynk.connected();

if (isconnected == false) {

wifiFlag = 1;

digitalWrite(wifiLed, HIGH); //Turn off WiFi LED

}

if (isconnected == true) {

wifiFlag = 0;

digitalWrite(wifiLed, LOW); //Turn on WiFi LED

}

}

void setup()

{

// Debug console

Serial.begin(115200);

Blynk.begin(auth, ssid, pass);

pinMode(ECHOPIN, INPUT);

pinMode(TRIGPIN, OUTPUT);

pinMode(RelayPin1, OUTPUT);

pinMode(RelayPin2, OUTPUT);

pinMode(RelayPin3, OUTPUT);

pinMode(RelayPin4, OUTPUT);

pinMode(wifiLed, OUTPUT);

pinMode(SwitchPin1, INPUT_PULLUP);

pinMode(SwitchPin2, INPUT_PULLUP);

pinMode(SwitchPin3, INPUT_PULLUP);

pinMode(SwitchPin4, INPUT_PULLUP);

//During Starting all Relays should TURN OFF

digitalWrite(RelayPin1, toggleState_1);

digitalWrite(RelayPin2, toggleState_2);

digitalWrite(RelayPin3, toggleState_3);

digitalWrite(RelayPin4, toggleState_4);

}

void loop()

{

//Blynk.run();

if (WiFi.status() != WL_CONNECTED)

{

Serial.println("WiFi Not Connected");

}

else

{

Serial.println("WiFi Connected");

Blynk.run();

//timer.run(); // Initiates SimpleTimer

}

if (wifiFlag == 0){

with_internet();

measureDistance();

}

else

without_internet();

delay(200);

}

void with_internet(){

//Manual Switch Control

if (digitalRead(SwitchPin1) == LOW){

delay(200);

relayOnOff(1);

Blynk.virtualWrite(VPIN_BUTTON_1, toggleState_1);   // Update Button Widget

}

else if (digitalRead(SwitchPin2) == LOW){

delay(200);

relayOnOff(2);

Blynk.virtualWrite(VPIN_BUTTON_2, toggleState_2);   // Update Button Widget

}

else if (digitalRead(SwitchPin3) == LOW){

delay(200);

relayOnOff(3);

Blynk.virtualWrite(VPIN_BUTTON_3, toggleState_3);   // Update Button Widget

}

else if (digitalRead(SwitchPin4) == LOW){

delay(200);

relayOnOff(4);

Blynk.virtualWrite(VPIN_BUTTON_4, toggleState_4);   // Update Button Widget

}

}

void without_internet(){

//Manual Switch Control

if (digitalRead(SwitchPin1) == LOW){

delay(200);

relayOnOff(1);

}

else if (digitalRead(SwitchPin2) == LOW){

delay(200);

relayOnOff(2);

}

else if (digitalRead(SwitchPin3) == LOW){

delay(200);

relayOnOff(3);

}

else if (digitalRead(SwitchPin4) == LOW){

delay(200);

relayOnOff(4);

}

}

void relayOnOff(int relay){

switch(relay){

case 1: if(toggleState_1 == 1){

digitalWrite(RelayPin1, LOW); // turn on relay 1

toggleState_1 = 0;

Serial.println("Device1 ON");

}

else{

digitalWrite(RelayPin1, HIGH); // turn off relay 1

toggleState_1 = 1;

Serial.println("Device1 OFF");

}

delay(100);

break;

case 2:

if(toggleState_2 == 1){

digitalWrite(RelayPin2, LOW); // turn on relay 2

toggleState_2 = 0;

Serial.println("Device2 ON");

}

else{

digitalWrite(RelayPin2, HIGH); // turn off relay 2

toggleState_2 = 1;

Serial.println("Device2 OFF");

}

delay(100);

break;

case 3:

if(toggleState_3 == 1){

digitalWrite(RelayPin3, LOW); // turn on relay 3

toggleState_3 = 0;

Serial.println("Device3 ON");

}

else{

digitalWrite(RelayPin3, HIGH); // turn off relay 3

toggleState_3 = 1;

Serial.println("Device3 OFF");

}

delay(100);

break;

case 4:

if(toggleState_4 == 1){

digitalWrite(RelayPin4, LOW); // turn on relay 4

toggleState_4 = 0;

Serial.println("Device4 ON");

}

else{

digitalWrite(RelayPin4, HIGH); // turn off relay 4

toggleState_4 = 1;

Serial.println("Device4 OFF");

}

delay(100);

break;

default : break;

}

}

// When App button is pushed - switch the state

BLYNK_WRITE(VPIN_BUTTON_1) {

toggleState_1 = param.asInt();

digitalWrite(RelayPin1, toggleState_1);

}

BLYNK_WRITE(VPIN_BUTTON_2) {

toggleState_2 = param.asInt();

digitalWrite(RelayPin2, toggleState_2);

}

BLYNK_WRITE(VPIN_BUTTON_3) {

toggleState_3 = param.asInt();

digitalWrite(RelayPin3, toggleState_3);

}

BLYNK_WRITE(VPIN_BUTTON_4) {

toggleState_4 = param.asInt();

digitalWrite(RelayPin4, toggleState_4);

}

void measureDistance(){

// Set the trigger pin LOW for 2uS

digitalWrite(TRIGPIN, LOW);

delayMicroseconds(2);

// Set the trigger pin HIGH for 20us to send pulse

digitalWrite(TRIGPIN, HIGH);

delayMicroseconds(20);

// Return the trigger pin to LOW

digitalWrite(TRIGPIN, LOW);

// Measure the width of the incoming pulse

duration = pulseIn(ECHOPIN, HIGH);

// Determine distance from duration

// Use 343 metres per second as speed of sound

// Divide by 1000 as we want millimeters

distance = ((duration / 2) * 0.343)/10;

Serial.println("Distance");

Serial.println(distance);

distance=30-distance;

Blynk.virtualWrite(V4, distance);

}